SECTION 09 90 00

PAINTS and COATINGS

MASTER

ARCHITECTURAL PAINTING SPECIFICATION

(Parallel Version – PPG-SW)

The information contained in this Behr Parallel Specification is primarily intended for use in reviewing commercial painting specification submittals. Because coating manufactures may change product formulations, users of this comparative data assume all responsibilities for acceptability of references such as resin types, composition, or gloss levels contained herein. Due to the many variables of Architect’s project painting specifications, it is recommended that the paint manufacturer(s) be contacted for their product’s intended use, technical data, SDS and VOC content compliance with authorities having jurisdiction.

These BEHR PAINTS are in conformance with LEED v4 BUILDING DESIGN AND CONSTRUCTION for LEED v4 BD+C New Construction, Core and Shell, Schools, Retail, Hospitality and Healthcare. All Behr Paints comply with South Coast AQMD Rule 1113. Interior and other Behr Paints are GREENGUARD® Gold Certified.

1801 E. St. Andrew Place, Santa Ana, California 92705 (714) 545-7101

**  NOTE TO SPECIFIER	This Section is for the specification of coatings in a mild to moderate environment.  It does contain some products from Diamond Vogel’s High Performance product lines to allow for design flexibility.  Diamond Vogel recommends the use of Section 09 96 00 High Performance Coatings for areas that require maximum protection.

##### PART 1 - GENERAL

##### 1.01 SUMMARY

Work includes furnishing of materials and equipment, preparation of surfaces and completion of painting and finishing of surfaces as required by Drawings and specified herein.

##### 1.02 RELATED WORK

- Factory, pre-finished items as specified in various sections.

- Shop painting specified in respective sections.

- Architectural woodworking.

- Surfaces not to be painted:

  - Pre-finished wall, ceiling and floor coverings.

  - Items with factory-applied final finish.

  - Concealed ducts, pipes and conduit.

  - Surfaces specifically scheduled or noted on Drawings not to be painted.

##### 1.03 SUBMITTALS

- Product data:

  - Not less than thirty (30) days before beginning work, submit a complete list of materials proposed for use, together with manufacturer's specifications.

  - Paint materials and products shall be subject to Architect's approval.

- Color samples:

  - Prepare color and finishes on samples, 8-1/2" x 11" in size.

  - Submit samples as requested until required sheen, color and texture is achieved.

  - Prepare wood samples on type and quality of wood specified for use on project.

  - Label and identify each sample as to location and application.

##### 1.04 COLORS

- Colors are to be selected or approved by Architect and actual color chips shall be supplied to Contractor for matching. All undercoats shall be tinted to approximately half the color of finish coat.

- Approval of final colors: Do not apply final coat of paint until colors have been approved by Architect.

- The number of colors to be used shall be as determined by Architect. Architect reserves right to vary colors throughout project.

##### 1.05 DELIVERY, STORAGE AND HANDLING

- Deliver paint materials in sealed original labeled containers bearing manufacturer's name, type of paint, stock number, color and instructions for reducing or mixing where applicable.

- Paint materials and equipment

  - Store only acceptable project materials on site.

  - Store in a suitable location.

  - Restrict storage to paint materials and related materials.

  - Comply with health and fire regulations.

##### 1.06 PROJECT CONDITIONS

- Comply with manufacturer's recommendations as to environmental conditions under which coatings and coating systems can be applied. Do not apply paint or coatings when temperature is below 50F. Do not apply exterior paint in damp or rainy weather; ensure that the surface has dried thoroughly before proceeding.  Surface temperature must be at least 5°F above dew point before painting.

- Do not apply finish in areas where dust or contaminants are being generated.

#### PART 2 - PRODUCTS

##### 2.01 MATERIALS

- Basis-of-Design Product: Provide products listed from Behr Paint Company, Santa Ana, California 92705, or comparable products by one of the following. Other manufacturers to conform to materials listed and be approved by Architect.

- PPG Industries, Inc. (PPG Paints)

- Sherwin-Williams Company (The)

- Materials selected for coating systems for each type surface shall be the product of a single manufacturer.

- Accessory materials such as thinner, linseed oil, putty and shellac shall be of the highest quality and by approved manufacturer, which comply with local VOC content limit regulations.

- Paints shall be ready-mixed except field catalyzed coatings.

- Do not thin finish coats without Architect's approval.

- Unsuitability of specified products: Claims concerning unsuitability of any material specified or inability to satisfactorily produce the work will not be entertained, unless such claim is made in writing to Architect before work is started.

- Number of coats scheduled is minimum. Apply additional coats at no additional cost if necessary to completely hide base materials, produce uniform color and provide satisfactory finish result.

- Submitted paints and coating shall comply with current local, state and federal air quality regulations.

#### PART 3 - EXECUTION

##### 3.01 EXAMINATION

- Examine surfaces scheduled to receive paint and finishes for conditions that will adversely affect execution, permanence or quality of work and which cannot be put into acceptable condition through preparatory work as included in Article 3.02 "Preparation ". Notify Owner’s Representative in writing of any defects or conditions which will prevent a satisfactory installation.

- Examine surfaces scheduled to be finished prior to commencement of work. Report any condition that may potentially effect proper application.

- Maximum Moisture Content of Substrates: When measured with an electronic moisture meter as follows;

  - Concrete: 12 percent

  - Portland Cement Plaster and Stucco: 12 percent

  - Masonry (Clay and CMU): 12 percent

  - Wood: 15 percent

  - Gypsum Board: 12 percent

- Portland Cement Plaster Substrates: Verify that plaster is fully cured.

- Verify suitability of substrates, including surface conditions and compatibility with existing finishes and primers.

- Proceed with surface preparation and coating application only after unsatisfactory conditions have been corrected.

- Application of coating is construed as acceptance of surfaces and conditions.

##### 3.02 PREPARATION

- Clean surfaces to receive paint thoroughly of substances, which could impair adhesion of paints, including dust, dirt, oil and grease before application of any coatings. Prepare surfaces as follows:

  - WOOD SUBSTRATES: Remove mill glaze and dust, sand smooth. Fill open joints, cracks, nail holes and other pits or depressions flush and smooth with wood filler after priming. Use wood putty to match finish paint coat. Touch up knots or sap streaks with a stain-blocking sealer before priming.

  - CONCRETE, PRE-CAST CONCRETE, TILT-UP: Remove release agents, curing compounds, loose particles, efflorescence and chalk.  Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - PLASTER and STUCCO SUBSTRATES: Fill hairline cracks, small holes and imperfections on surfaces with patching compound. Smooth off to match adjacent surfaces. Apply an alkali-resistant primer or wash with fresh water and neutralize high alkalinity surfaces where they occur.

  - MASONRY SUBSTRATES: Remove efflorescence and chalk. Do not coat surfaces if moisture content or alkalinity of surface to be coated exceeds that permitted in the manufacturer’s written instructions. Prime with an alkali-resistant primer.

  - SHOP-PRIMED STEEL SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose primer and rust, contaminants and foreign matter. Prime bare steel and touch up abrasions with a ferrous metal primer.

  - STEEL, FERROUS METAL SUBSTRATES: Remove rust, mill scale, foreign substances and shop primer. Clean according to SSPC-SP3, “Power Tool Cleaning”, or abrasive blasting cleaning according to SSPC-SP6 “Commercial Blast Cleaning” as required. Protect surface from corrosion until application of primer.

  - GALVANIZED METAL: Remove oils, passivators and clean entire surface with an appropriate solvent. Pre-treat with a phosphoric acid etching solution to promote adhesion of subsequently applied coatings.

  - PHOSPHATIZED METAL or GALVANNEALED METAL shall not be chemically etched. Clean and apply suitable metal primer.

  - ALUMINUM SUBSTRATES: Solvent clean according to SSPC-SP1. Remove loose surface oxidation.

  - GYPSUM BOARD SUBSTRATES: Remove dust and foreign matter. Fill pits flush and smooth with joint compound and where required, apply skim coat to provide the required finishing level based on GA-214-96, Recommended Levels of Gypsum Board Finish before application of decoration.

- Surfaces, which cannot be prepared or painted as specified, shall be immediately brought to attention of Architect in writing.

  - Starting of work without such notification will be considered acceptance by the Contractor of surfaces involved.

  - Replace unsatisfactory work caused by improper or defective surfaces as directed by Architect at no additional cost to Owner.

##### 3.03 APPLICATION

- Do not apply initial coating until moisture content of surface is within limitations recommended by paint manufacturer.

- Application:

  - Apply paint with suitable brushes, rollers or spraying equipment.

  - Apply stain in accordance with manufacturer's recommendations.

  - Rate of application shall not exceed that as recommended by paint manufacturer for surface involved.

- Comply with recommendations of product manufacturer for drying time between succeeding coats.

- Leave parts of molding and ornaments clean and true to details with no undue amount of paint in corners and depressions.

- Make edges of paint adjoining other material or color clean and sharp with no overlapping.

- Refinish whole wall where portion of finish is not acceptable.

- Apply materials evenly with proper film thickness and free of runs, sags, skips and other defects. Hard, glossy finishes shall be sanded lightly between coats, dusted and cleaned before recoating.

- Remove hardware, hardware accessories, plates, lighting fixtures and similar items in place prior to painting and replaced upon completion of each space.

- Disconnect heating and other equipment adjacent to walls using workmen skilled in appropriate trades, and moved to permit wall surfaces to be painted. Following completion of painting, they shall be expertly replaced and reconnected.

- Paint visible surfaces behind vents, registers or grilles flat black.

  - Wash exposed metal with solvent then prime and paint as scheduled.

  - Spray paint wherever practical.

- Do not paint over Underwriters' labels, fusible links or sprinkler heads.

- EXPOSED PLUMBING AND MECHANICAL ITEMS: Items without factory finish such as conduits, pipes, access panels and items of similar nature shall be finished to match adjacent wall and ceiling surfaces unless otherwise directed.

##### 3.04 CLEANUP

Upon completion of work, remove equipment, excess material and debris, remove paint splatters and leave area in a neat and orderly condition.

##### 3.05 PROTECTION

- Protect work of other trades, whether to be painted or not, against damage by painting. Correct damage by cleaning, repairing or replacing, and repainting, as acceptable to Architect.

- Provide "wet paint" signs to protect newly painted finishes. Remove temporary protective wrappings provided by others for protection of their work after completion of painting operations.

  - At completion of construction activities of other trades, touch up and restore damaged or   defaced painted surfaces.

##### 3.06 FINISH SCHEDULE

- Finish surfaces in accordance with schedule. Catalog names and numbers refer to products as manufactured by the Behr Paint Company, Santa Ana, California 92705, except as otherwise specified.

- Provide paint finishes of even uniform color, free from cloudy or mottled appearance. Properly correct non- complying work to satisfaction of Owner’s representative and representative of Behr Process Corporation.

- Some colors, especially accent colors may require multiple finish coats for adequate coverage and opacity.

- The specified number of primer and finish coats is minimum acceptable. If full coverage and opacity is not obtained with specified number of coats, apply additional coats as necessary to produce required finish.

- Application Finish Schedule:

EXTERIOR PAINT SCHEDULE:

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Stucco, Cement Plaster

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 |
| Topcoat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |

Flat – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Concrete Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Exterior Flat 6-610XI | A-100 Exterior Flat A6-100 Series |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Exterior Flat 6-610XI | A-100 Exterior Flat A6-100 Series |

Satin – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Concrete Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Concrete Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Ext. Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |
| Topcoat: | Behr Pro e600 Ext. Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |

- Stucco and Cement Plaster – High Build Coating

Flat – Exterior High Build Coating 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Concrete Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |
| Topcoat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |

- Stucco, Cement Plaster, Masonry & Brick - Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |

4.	Concrete, Concrete Pre-Cast, Concrete Tilt-Up

Flat – Exterior, Professional, 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Premium Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Concrete, Concrete Pre-Cast, Concrete Tilt-Up – High Build Coating

Flat – Exterior High Build Coating, 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |
| Topcoat: | Premium Exterior High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |

- Concrete, Concrete Pre-Cast, Concrete Tilt-Up – Elastomeric

Exterior 100% Acrylic Elastomeric Coating

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |

- Concrete Masonry Units (CMU)

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |
| Topcoat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A6-100 / A-100 Exterior Flat |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A6-100 / A-100 Exterior Flat |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | A-100 Latex Gloss A8-100 |

- Concrete Masonry Units (CMU) - High Build Coating

Flat – High Build Coating 100% Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |
| Topcoat: | Premium High Build Coating 4700 | Perma-Crete High Build Acrylic 4-22 | Loxon XP High Build Coating LX11W0051 |

- Concrete Masonry Units (CMU) - Elastomeric

Exterior 100% Acrylic Elastomeric Coating,

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W2524 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |
| Topcoat: | Premium Exterior Elastomeric Paint 68 | Pitt-Flex Elastomeric 4-110 P-C | Conflex SherLastic CF16W0051 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Concrete Porches and Patios

Low Luster Exterior/Interior 100% Acrylic

| Prime Coat: | Concrete Bonding Primer 880* | None recommended | Tread-Plex Primer B90W110 |
| --- | --- | --- | --- |
| Intermediate Coat: | Porch/Patio Low Lustre Enamel 6050 | Floor & Porch Satin Latex 3-510 | Porch & Floor Satin A32-200 Series |
| Topcoat: | Porch/Patio Low Lustre Enamel 6050 | Floor & Porch Satin Latex 3-510 | Porch & Floor Satin A32-200 Series |

*Note: Intended for use on rough and porous concrete surfaces.

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | N/A | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6705 | N/A | N/A |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6705 | N/A | N/A |

Note* Intended for use on rough and porous concrete surfaces.

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Solid Color Coating

Flat – Premium Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr Premium DeckOver 5000 | N/A | SuperDeck Deck & Dock SD9W00151 |
| --- | --- | --- | --- |
| Topcoat: | Behr Premium DeckOver 5000 | N/A | SuperDeck Deck & Dock SD9W00151 |

- Concrete, Decks, Walkways, Pool Decks and Sidewalks – Textured Solid Color Coating

Flat – Premium Exterior Solid Color Coating, 100% Acrylic

| Prime Coat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |
| Topcoat: | Behr Textured DeckOver Solid Color Coating 5005 | N/A | N/A |

- Wood, Paint Finish

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |
| Topcoat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | 17-921 Seal Grip Primer & Sealer | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Wood, Paint Finish – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Seal Grip Primer & Sealer 17-921 | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Seal Grip Primer & Sealer 17-921 | PrepRite ProBlock B51-600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Int/Ext Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Alkyd Int/Ext Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Semi-Gloss B53-1250 |

- Wood, Stain Finish, Water-Based, Semi-Transparent

Flat – 100% Acrylic

| Two Coats: | Semi-Transparent Stain, Sealer 5077 | Sun Proof Ext. S/T Stain 77-1460 | A15T WoodScapes S/T Stain |
| --- | --- | --- | --- |
| Or - Two Coats: | Redwood S/T Stain 5330 |  |  |
| Or – Two Coats | Cedar Naturaltone S/T Stain 5533 |  |  |

- Wood, Stain Finish, Water-Based, Solid Color

Flat – 100% Acrylic

| Two Coats: | Solid Color Stain White 5011 | Sun Proof Solid Color 77-1310 | Acrystain CK6688 Series |
| --- | --- | --- | --- |
| Or - Two Coats: | Solid Color Deep Base 5013 |  |  |
| Or – Two Coats | Cedar Naturaltone S/T Stain 50533 |  |  |

- Wood, Wood Composite – Decks, Patios, Boat Docks, Railings – Solid Color Coating

Flat – 100% Acrylic, Solid Color Coating

| Two Coats: | DeckOver Solid Color Coating 5000 | N/A | SuperDeck Deck & Dock SD9W00151 |
| --- | --- | --- | --- |

- Wood, Wood Composite – Decks, Patios, Boat Docks, Railings – Texture Solid Color Coating

Flat – 100% Acrylic, Solid Color Coating

| Two Coats: | DeckOver Texture Coating 5005 | N/A | N/A |
| --- | --- | --- | --- |

- Steel, Ferrous Metal

Flat – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |
| Topcoat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |

Satin – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |

Semi-Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |

Gloss – Premium Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Flat – Exterior, Professional 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |

Satin – Exterior, Professional, 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional, Exterior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66-1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66-1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Seal Grip Primer & Sealer 17-921 | ProCryl Universal Primer B66-1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Alkyd Int/Ext Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Alkyd Int/Ext Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

- Steel, Ferrous Metal – High Performance, Water-Based, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |
| Topcoat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |
| Topcoat: | Premium Plus Exterior Flat 4050 | Manor Hall Exterior Flat 70-101 | SuperPaint Flat A80-1100 Series |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Eggshell (Satin) – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |
| Topcoat: | Premium Plus Exterior Satin 9050 | Sun Proof Satin Latex 76-45 | SuperPaint Ext. Satin A89-1100 |

Semi-Gloss – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |
| Topcoat: | Premium Plus Exterior Semi-Gloss 5050 | Sun Proof Semi-Gloss Latex 78-45 | SuperPaint Ext. Gloss A84-1100 |

Gloss – Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Int/Ext Gloss 52-110 | SuperPaint Ext. High Gloss A85 |

Flat – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |
| Topcoat: | Behr Pro e600 Exterior Flat 610 | Speedhide Acrylic Flat 6-610XI | A-100 Exterior Flat A6-100 Series |

Satin – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |
| Topcoat: | Behr Pro e600 Exterior Satin 640 | Speedhide Ext. Satin 6-2045XI | A-100 Exterior Satin A82-100 |

Semi-Gloss – Professional Exterior 100% Acrylic

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |
| Topcoat: | Behr Pro e600 Exterior Semi-Gloss 670 | Speedhide Ext. Semi-Gloss 6-900XI | Solo Semi-Gloss 76 Series |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Interior / Exterior Enamel

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66W1310 |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |

- Galvanized Metal, Aluminum, Non-Ferrous Metals – High Performance, Industrial Maintenance

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Klean Strip Prep & Etch GKPA30220 | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Pro-Cryl Universal Primer B66-W01310 |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |
| Topcoat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |

INTERIOR PAINT SCHEDULE

- Gypsum Board

Flat – Premium Interior, Acrylic

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Harmony Primer B11W01500 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Harmony Primer B11W01500 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300XI | Harmony Eg-Shel B09-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300XI | Harmony Eg-Shel B09-1000 |

Satin – Premium Interior, Acrylic

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Harmony Primer B11W01500 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500 | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500 | Harmony Semi-Gloss B10-1000 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | Speedhide Zero Interior Latex Sealer 6-4900XI | ProMar 200 Zero VOC Primer B28W02600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | Speedhide Zero Interior Latex Sealer 6-4900XI | ProMar 200 Zero VOC Primer B28W02600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20W2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20W2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Kilz PVA Interior Primer 10 | Speedhide Zero Interior Latex Sealer 6-4900XI | ProMar 200 Zero VOC Primer B28W02600 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar Zero VOC Semi-Gloss B31-2600 |

- Gypsum Board: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

- Gypsum Board – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide Water-Borne Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Premium Plus Drywall Primer Sealer 73 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W8111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

- Gypsum Board - Ceilings

Flat – Interior, Acrylic

| Prime Coat: | Kilz Pro PVA Drywall Primer 10 | Speedhide Latex Sealer 6-2 | PVA Primer Sealer B28W8000 |
| --- | --- | --- | --- |
| Intermediate Coat: | Interior Ceiling Flat 5558 | Ceiling Paint Flat Latex 17-45 | Eminence Ceiling Flat A27 |
| Topcoat: | Interior Ceiling Flat 5558 | Ceiling Paint Flat Latex 17-45 | Eminence Ceiling Flat A27 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Gypsum Board - Dry Fall Coating (Spray Applied)

Flat – Interior, Acrylic, Spray Applied

| Prime Coat: | Kilz Pro PVA Drywall Primer 10 | Speedhide Latex Sealer 6-2 | PVA Primer Sealer B28W8000 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Dry Fall Flat B42 Series |
| Topcoat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Dry Fall Flat B42 Series |

- Concrete, Plaster, Masonry

Flat – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |

Satin – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Concrete, Plaster, Masonry: Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus All-In-One Primer 75 | Pure Performance Primer 9-900 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

- Concrete, Plaster, Masonry – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

- Concrete, Plaster, Masonry – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Satin – Interior / Exterior Enamel

| Prime Coat: | All-In-One Primer Sealer 75 | Perma-Crete Primer 4-603 | Loxon Primer A24W8300 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

- Concrete Porches and Patios

Low Luster Exterior/Interior 100% Acrylic

| Prime Coat: | 880 Concrete Bonding Primer* | N/A | Tread-Plex Primer B90W110 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Enamel 6050 | Floor & Porch Satin Latex 3-510 | Porch & Floor Satin A32-200 Series |
| Topcoat: | Premium Porch/Patio Enamel 6050 | Floor & Porch Satin Latex 3-510 | Porch & Floor Satin A32-200 Series |

*Note: Intended for use on rough and porous concrete surfaces.

Gloss Exterior/Premium, Latex, Interior, Low Odor/VOC

| Prime Coat: | Concrete & Masonry Bonding Primer 880* | Floor & Porch WB Gloss 3-610 | N/A |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Porch/Patio Gloss Enamel 6700 | Floor & Porch WB Gloss 3-610 | N/A |
| Topcoat: | Premium Porch/Patio Gloss Enamel 6700 | Floor & Porch WB Gloss 3-610 | N/A |

- Concrete Masonry Units (CMU)

Flat – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Satin – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 2-8050 | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |

- Concrete Masonry Units (CMU): Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Concrete Masonry Units (CMU) – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Interior / Exterior Enamel

| Prime Coat: | Behr Pro Block Filler Primer 50 | Speedhide Block Filler 6-15 | Block Filler B25W25 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1510 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1510 | WB Alkyd Urethane Low Sheen B53-1250 |

- Steel, Ferrous Metal

Flat – Premium Interior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |

Satin – Premium Interior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Eggshell 9-300Xl | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Eggshell 9-300Xl | Harmony Semi-Gloss B10-1000 |

Gloss – Premium Interior 100% Acrylic

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050* | Pure Performance Semi-Gloss 9-500Xl | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050* | Pure Performance Semi-Gloss 9-500Xl | Solo Gloss A77 Series |

(Note: 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |

- Steel, Ferrous Metal – Light Industrial Coatings

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

- Steel, Ferrous Metal – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Premium Interior / Exterior Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

- Ferrous Metal – High Performance, Industrial Maintenance

Semi-Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |
| Topcoat: | Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Gloss, 100% Acrylic, Direct-To-Metal Enamel

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |
| Topcoat: | Direct-To-Metal Gloss 8200 | Pitt-Tech Plus Gloss DTM 90-1310 | Pro Industrial DTM Gloss B66W01051 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals

Flat –Premium Interior 100% Acrylic

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior 100% Acrylic

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | Pure Performance Eggshell 9-300Xl | Harmony Eg-Shel B09-1000 |

Satin – Premium Interior 100% Acrylic

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior 100% Acrylic

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |

Gloss – Premium Interior 100% Acrylic

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050* | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050* | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |

(Note: 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Light Industrial Coatings

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Acrylic Metal Primer M04 |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

- Galvanized Metal, Aluminum, and Non-Ferrous Metals – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Enamel

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

Satin – Premium Interior / Exterior Enamel

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Semi-Gloss 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

- Galvanized Metal, Aluminum,  Non-Ferrous Metals – High Performance, Industrial Maintenance

Semi-Gloss, Premium 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |
| Topcoat: | Premium Direct-To-Metal Semi-Gloss 3200 | Pitt-Tech Plus Semi-Gloss DTM 90-1210 | Pro Industrial DTM Semi-Gloss B66W01151 |

Gloss, Premium 100% Acrylic, Direct-To-Metal Enamel

| Pretreatment: | Krud Kutter Metal Clean & Etch | No recommendation | Krud Kutter Metal Cleaner & Etch |
| --- | --- | --- | --- |
| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | ProCryl Universal Primer B66W01310 |
| Intermediate Coat: | Direct-To-Metal Gloss 8200 | 90-1310 Pitt-Tech Plus Gloss DTM | Pro Industrial DTM Gloss B66W01051 |
| Topcoat: | Direct-To-Metal Gloss 8200 | 90-1310 Pitt-Tech Plus Gloss DTM | Pro Industrial DTM Gloss B66W01051 |

- Galvanized Metal and Non-Ferrous Metals – Dry Fall Coating (Spray applied)

Flat, Acrylic, Dry Fall

| Prime Coat: | Premium Plus Multi-Surface Primer 436 | Pitt-Tech Plus Primer 90-912 | Multi-Purpose Primer B51-450 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Dry Fall Flat B42 Series |
| Topcoat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Dry-Fog Flat 6-275Xl | Dry Fall Flat B42 Series |

- WOOD – Paint Finish

Flat – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Flat 1050 | Pure Performance Flat 9-100 | Harmony Flat B05-1000 |

Eggshell – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Eggshell 2050 | 9-300XI Pure Performance Eggshell | Harmony Flat B05-1000 |
| Topcoat: | Premium Plus Interior Eggshell 2050 | 9-300XI Pure Performance Eggshell | Harmony Flat B05-1000 |

Satin – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |
| Topcoat: | Premium Plus Interior Satin 7050 | Manor Hall Satin 82-400 | Super Paint Satin A87-1100 |

Semi-Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |
| Topcoat: | Premium Plus Interior Semi-Gloss 3050 | Pure Performance Semi-Gloss 9-500Xl | Harmony Semi-Gloss B10-1000 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Gloss – Premium Interior, Acrylic

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Premium Plus Int/Ext Hi-Gloss 8050* | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |
| Topcoat: | Premium Plus Int/Ext Hi-Gloss 8050* | Manor Hall Gloss 52-110 | Solo Gloss A77 Series |

(Note: 2-8050 Hi-Gloss Enamel conforms to VOC limits of South Coast AQMD Rule 1113.)

Flat – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |
| Topcoat: | Behr Pro i300 Interior Flat Paint 310 | Speedhide Zero Interior Flat 6-4110 | ProMar 200 Zero VOC Flat B30-2600 |

Eggshell – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |
| Topcoat: | Behr Pro i300 Interior Eggshell 330 | Speedhide Zero Interior Eggshell 6-4310XI | ProMar 200 Zero VOC Eg-Shel B20-2600 |

Semi-Gloss – Professional, Latex, Interior, Low Odor/VOC

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |
| Topcoat: | Behr Pro i300 Interior Semi-Gloss 370 | Speedhide Zero Interior Semi-Gloss 6-4510XI | ProMar 200 Zero VOC Semi-Gloss B31-2600 |

- WOOD – Light Industrial Coating

Eggshell – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Eggshell HP140 | Pitt-Glaze 16-310 WB Eggshell Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K45 -150 Eggshell |

Semi-Gloss – Interior Pre-Catalyzed Waterborne Epoxy

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |
| Topcoat: | Behr Pro Pre-Catalyzed Waterborne Epoxy Semi-Gloss HP150 | Pitt-Glaze 16-510 WB Semi-Gloss Pre-Catalyzed Water-Borne Acrylic Epoxy | Pro Industrial Pre-Catalyzed Waterbased Epoxy K46 -150 Semi-Gloss |

- WOOD – Paint Finish – Urethane Alkyd, Water-Reducible and Alkyd Waterborne

Semi-Gloss – Premium Interior / Exterior Alkyd Enamel

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |
| Topcoat: | Urethane Alkyd Semi-Gloss Enamel 3900 | Speedhide WB Alkyd Semi-Gloss 6-1510 | WB Alkyd Urethane Semi-Gloss B53-1150 |

| BEHR | PPG Paints | Sherwin-Williams |
| --- | --- | --- |

Satin – Premium Interior / Exterior Enamel

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |
| Topcoat: | Urethane Alkyd Satin Enamel 7900 | Speedhide WB Alkyd Satin 6-1410 | WB Alkyd Urethane Low Sheen B53-1250 |

- WOOD – Paint Finish – Dry Fall Coating (Spray applied)

Flat, Acrylic, Dry Fall

| Prime Coat: | Premium All-In-One Primer Sealer 75 | Pure Performance Primer 9-900 | Premium Wall & Wood Primer B28W08111 |
| --- | --- | --- | --- |
| Intermediate Coat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Super Tech WB Dry-Fog Flat 6-725 | Dry Fall Flat B42 Series |
| Topcoat: | Behr Pro Dryfall Flat 890 White, 891 Black | Speedhide Super Tech WB Dry-Fog Flat 6-725 | Dry Fall Flat B42 Series |

END OF SECTION 09 90 00